Link: https://wokwi.com/projects/393180528527977473

Sửa file User_Setup.h

46 - #define ST7735_DRIVER

76 - #define TFT_RGB_ORDER TFT_RGB

85 - #define TFT_WIDTH  128

88 - #define TFT_HEIGHT 160

109 - #define ST7735_REDTAB

Thêm:

// For ESP32 Dev board (only tested with ST7735 display)

// The hardware SPI can be mapped to any pins

#define TFT_MOSI 23 // In some display driver board, it might be written as "SDA" and so on.

#define TFT_SCLK 18

#define TFT_CS   5  // Chip select control pin

#define TFT_DC   32  // Data Command control pin

#define TFT_RST  15  // Reset pin (could connect to Arduino RESET pin)

//#define TFT_BL   22  // LED back-light

#define LOAD_GLCD   // Font 1. Original Adafruit 8 pixel font needs ~1820 bytes in FLASH

#define LOAD_FONT2  // Font 2. Small 16 pixel high font, needs ~3534 bytes in FLASH, 96 characters

#define LOAD_FONT4  // Font 4. Medium 26 pixel high font, needs ~5848 bytes in FLASH, 96 characters

#define LOAD_FONT6  // Font 6. Large 48 pixel font, needs ~2666 bytes in FLASH, only characters 1234567890:-.apm

#define LOAD_FONT7  // Font 7. 7 segment 48 pixel font, needs ~2438 bytes in FLASH, only characters 1234567890:-.

#define LOAD_FONT8  // Font 8. Large 75 pixel font needs ~3256 bytes in FLASH, only characters 1234567890:-.

//#define LOAD_FONT8N // Font 8. Alternative to Font 8 above, slightly narrower, so 3 digits fit a 160 pixel TFT

#define LOAD_GFXFF  // FreeFonts. Include access to the 48 Adafruit_GFX free fonts FF1 to FF48 and custom fonts

// Comment out the #define below to stop the SPIFFS filing system and smooth font code being loaded

// this will save ~20kbytes of FLASH

#define SMOOTH_FONT

-------------------------------

#define SPI_FREQUENCY  27000000

#define SPI_READ_FREQUENCY  20000000

#define SPI_TOUCH_FREQUENCY  2500000