Hướng dẫn về thư viện TFT_eSPI

Link tham khảo:

https://doc-tft-espi.readthedocs.io/starting/

https://randomnerdtutorials.com/esp32-tft-touchscreen-display-2-8-ili9341-arduino/

https://www.instructables.com/Using-TFTeSPI-Library-With-Visual-Studio-Code-and-/

Search google:

To use the TFT_eSPI library with an ESP32, you need to install the library, configure a specific User_Setup.h file for your display hardware and wiring, and then use the library's functions in your code.

Step 1: Hardware & Wiring

First, identify your TFT display's driver chip (e.g., ILI9341, ST7789) and resolution, which can often be found on the product page or datasheet. The ESP32 communicates with the display using the SPI protocol, so ensure your connections use the hardware SPI pins for better performance.

Common SPI pins for the ESP32 include:

- MOSI: GPIO 23

- MISO: GPIO 19

- SCK: GPIO 18

You will also need GPIO pins for Chip Select (CS), Data/Command (DC), and Reset (RST). Ensure your display's backlight (BL) pin is connected to an appropriate power source or GPIO pin if it requires control.

Step 2: Library Installation

You can install the library using the Arduino IDE Library Manager:

- Open the Arduino IDE and go to Sketch > Include Library > Manage Libraries.

- Search for "TFT_eSPI" and install the library by Bodmer.

Random Nerd Tutorials

Step 3: Library Configuration

This is the most critical step, as the library must be configured to match your specific hardware and pinout.

For Arduino IDE:

- Navigate to your Arduino sketchbook location and open the libraries folder.

- Open the TFT_eSPI folder and locate the User_Setup.h configuration file.

- Replace this default file with a custom User_Setup.h file that matches your specific board and wiring. Many tutorials, like those on Random Nerd Tutorials or Makerguides.com, provide pre-configured files for common setups.

- Alternatively, you can edit the existing User_Setup.h file manually, commenting out other drivers and defining only your specific display driver and pin assignments using #define statements.

For PlatformIO (VS Code):
Instead of modifying the library files directly (which can be overwritten), use build_flags in your platformio.ini file to define the configuration macros.

ini

build_flags =

-D USER_SETUP_LOADED

-D ILI9341_DRIVER  ; Replace with your driver (e.g., ST7789_DRIVER)

-D TFT_MISO=19

-D TFT_MOSI=23

-D TFT_SCLK=18

-D TFT_CS=15

-D TFT_DC=2

-D TFT_RST=4

; Add other specific settings like fonts, backlight pin, etc.

For a detailed guide on using PlatformIO, refer to tutorials available on platforms like Level Up Coding.

Step 4: Example Code

Once configured, a minimal program to test your setup looks like this:

cpp

#include <TFT_eSPI.h>

TFT_eSPI tft = TFT_eSPI();

void setup() {

tft.init();             // Initialize the display

tft.setRotation(1);     // Optional: Rotate the screen (0-3)

tft.fillScreen(TFT_BLACK); // Fill background

tft.setCursor(0, 0, 4); // Set cursor to top-left, font size 4

tft.setTextColor(TFT_WHITE); // Set text color

tft.println("Hello World!"); // Display text

}

void loop() {

// Your main code here

}

If your screen doesn't light up, double-check your wiring and User_Setup.h configuration. The library's GitHub repository and examples folder contain many more advanced examples, including drawing shapes, using fonts, and sprites.

Instructables +1